Icon Updating

- Hold pressing PTT key and PF2 key (the bottom side key), at the same time power on radio, LCD display UPDATE MODE

- Use PC cable connect PC to radio

- In CPS-> Set ->Set Com, choose the correct Com Port.

- In CPS ->Tool, choose the firmware upgrade. Or use the

- QX_Firmware_Update.exe for firmware upgrade

- Click Open Update File，choose D878_1G_ICON_V1.1b_20180818.spi

- Com Speed  921600, tick on Duplex

- Click Write to update the radio the latest firmware.

- Reboot the radio when the update is finished.